What is blood pressure?

It’s the measurement of the force and pressure that your heart pumps to the arteries, and its measured as two numbers, systolic and diastolic
Systolic: pressure when heart pumps

Diastolic: pressure when heart at rest between two pumps

Blood pressure changes based on a few things:

- Age

- Your status (resting or exercising)

- Medicine and food

- Your position (sitting, standing)

What’s happening during a blood pressure test:

- As the blood pressure cuff inflates, you'll feel it tighten around your arm. This blocks the blood flow in that part of your arm for a short time.

- Your provider will then open a valve on the cuff to slowly release air. As the cuff deflates, blood flow will start again in your arm.

- As the pressure falls, a measurement is taken when the sound of blood pulsing is first heard. This is the systolic pressure.

- As the air continues to be let out, the blood pulsing sound will start to go away. When it completely stops, another measurement is taken. This is the diastolic pressure.

Normal blood pressure levels:

| Blood Pressure Category | Systolic Blood Pressure |  | Diastolic Blood Pressure |
| --- | --- | --- | --- |
| Normal | Less than 120 | and | Less than 80 |
| Elevated | 120 - 129 | and | Less than 80 |
| High Blood Pressure Stage 1 | 130 - 139 | or | 80 - 89 |
| High Blood Pressure Stage 2 | 140 or higher | or | 90 or higher |
| Hypertensive Crisis (dangerously high blood pressure - seek medical care right away | Higher than 180 | and | Higher than 120 |

MATERIALS:

- Arduino UNO  450EGP

- I2C Serial Interface   60EGP

- 16x2 LCD  90EGP

- Pressure Sensor  110egp

- Servo Controlled Valve  120EGP

- Air pump   180egp

- (4) 100Kohm resistors   5EGP

- (1) 1Kohm resistor   5EGP

- (2) 1uF Capacitors   5EGP

- PVC Pipe as 3 way splitter   80EGP

- Tubing   20EGP

- Blood Pressure Cuff And Manual Pump   200EGP

- TL072 Op-Amp    8EGP

- Bluetooth Module   245EGP

- pulse oximeter    110EGP

- Breadboard(s)   100EGP

- Wires

- 9V Battery   40EGP

- 5V Battery   21EGP

Total Price: 1849 EGP

-High pass filter (2Hz cutoff) and Low pass filter (10Hz cutoff)